RECORD-INFORMED SINUSOIDAL SURROGATE LOADING FOR THE DISTINCT ELEMENT SIMULATION OF A SEQUENTIAL OUT-OF-PLANE SHAKE-TABLE TEST ON AN UNREINFORCED MASONRY WALL

Yopi P. Oktiovan1, Bora Pulatsu2, Nicolò Damiani3,4, Eleni Smyrou1 and Ihsan E. Bal1

1 Research Centre NoorderRuimte, Hanze University of Applied Sciences, Groningen, the Netherlands

2 Department of Civil and Environmental Engineering, Carleton University, Ottawa, Canada

3 Department of Civil Engineering and Architecture (DICAr), University of Pavia, Pavia, Italy

4 European Centre for Training and Research in Earthquake Engineering (EUCENTRE), Pavia, Italy

e-mail: [corresponding e-mail]

Keywords: Distinct Element Method, unreinforced masonry, out-of-plane, shake-table sequence, surrogate loading, mean period.

Out-of-plane shake-table campaigns on unreinforced masonry (URM) walls often subject a single specimen to tens of scaled records of increasing intensity. To reproduce such a campaign with the Distinct Element Method (DEM), the whole sequence must be run on one progressively damaged model, at considerable computational cost. A short sinusoidal signal of equivalent spectral demand could replace each record at a fraction of that cost, provided it carries both the demand on the wall and the frequency content of the record.

A surrogate of this kind is developed here for a 25-run sequence on a solid two-leaf URM wall (1.29 × 2.58 × 0.21 m) with timber floor joists, two specimens of which were tested out of plane under scaled HU12, EC40 and FR76 records [1]. Two single-cycle sine waves of table velocity stand in for each record. Their amplitudes are governed by the wall, fitted jointly until the 5%-damped displacement spectrum of the combined signal equals that of the scaled record at two periods. These are the elastic period T1 and an effective period Teff, obtained once from the numerical pushover curve and the measured effective mass and held constant over the sequence. The periods of the sine waves, in contrast, are read from the record alone. The first wave is set at the predominant period Tp, and the second at the mean period Tm [2], a Fourier-amplitude-weighted average of the record’s frequency content that equals 0.21, 0.54 and 0.37 s for HU12, EC40 and FR76.

The pairing follows from a signal-level comparison of candidate carrier periods, made before any DEM analysis. With both waves at Tm, the fit degenerates into a single wave and the Teff target is overshot by as much as 76%. Placing both at Tp drives the table acceleration up to 3.6 times the record PGA. Of the combinations considered, only Tp/Tm matches both spectral ordinates on all 25 runs within 4% of the scaled record PGA.

In 3DEC, the wall is modelled as an assembly of rigid blocks whose joints follow the MASON contact model [3], with zero tensile strength as indicated by the force plateau in the tests, under large-strain kinematics and 1.5% Maxwell damping. Run by run, the surrogate response is compared with the same model under the recorded table motions and with the two specimens, in terms of peak and residual out-of-plane displacement, peak base shear and force–displacement hysteresis. Both spectral targets are reproduced in 3DEC within 0.4%. Over the first 15 runs, the peak base shear stays within 0.84–1.33 times that measured on the two specimens. Peak displacement agrees with the record-driven model within 20% up to run 7, but is overestimated by up to 2.3 times by run 15. In those runs the wall still responds at 0.16–0.20 s, while the second wave imposes the demand at Teff = 0.34 s.

A sensitivity analysis, presented alongside these results, examines how the response depends on the ultimate displacement that fixes Teff, the damping, the number of cycles and relative phase of the two waves, and the pushover branch used for T1 and Teff. The results are expected to define the conditions under which a record-informed two-pulse surrogate can replace a full record sequence in DEM analyses of out-of-plane loaded URM walls, and the range of validity of its single-degree-of-freedom basis.

References

[1]  Smyrou E, Moshfeghi A, Arslan O, Bal IE (2024). Out-of-plane shake table tests on solid masonry walls with timber floors. Structures, 66, 106815.

[2]  Rathje EM, Faraj F, Russell S, Bray JD (2004). Empirical relationships for frequency content parameters of earthquake ground motions. Earthquake Spectra, 20(1), 119–144.

[3]  Oktiovan YP, Messali F, Pulatsu B, Sharma S, Lemos JV, Rots JG (2026). Cyclic constitutive model for masonry joint damage and energy dissipation using the distinct element method. Computers and Structures, 321, 108094.